Interface to Iridium Subscriber Unit

| Pin | Signal | Description |
| --- | --- | --- |
| 1 | ISU_SENSE_11HZ | Reserved |
| 2 | EXT_ON_OFF | Power on/off from external peripherals |
| 3 | DSC_EN_IGS | DSC bus enable and IGN sense |
| 4 | DOWNLINK | DSC bus downlink |
| 5 | UPLINK | DSC bus uplink |
| 6 | CHRG_EN | Reserved |
| 7 | NC | No connection |
| 8 | S_TX | Data transmit (RS-232) |
| 9 | EXT_B+ (4.4V) | External power (B+) |
| 10 | S_RX | Data receive (RS-232) |
| 11 | MAN_TEST | Reserved |
| 12 | GND | Ground |
| 13 | BATT_FDBK | Reserved |
| 14 | NC | No connection |
| 15 | RI | Ring indicator (RS-232) |
| 16 | DTR | Data terminal ready (RS-232) |
| 17 | RTS | Request to send (RS-232) |
| 18 | DSR | Data set ready (RS-232) |
| 19 | DCD | Data carrier detect (RS-232) |
| 20 | CTS | Clear to send (RS-232) |

# Below is pulled from LBT Interface Specification, but applies equally to an ISU since they have the same internal circuit board.

# External On/Off

A peripheral can turn the LBT (or ISU) ON and OFF using the EXT_ON_OFF line.  The LBT will automatically power-up when external power is supplied.  (Not necessarily true for ISU)  After initial EXT_B+ is applied, each subsequent toggle of the EXT_ON_OFF line will result in changing the current state of the LBT, OFF to ON or ON to OFF.  If the signal is pulsed too quickly when the LBT is ON, the LBT may remain ON.  The timing and results of the EXT_ON_OFF signal for the LBT is shown below.

| LBT Start State | # EXT_ON_OFF | LBT End State |
| --- | --- | --- |
| OFF | Pulse (drive low, release high) | ON |
| ON | Pulse | ON |
| OFF | Hold (hold low>270ms) | ON |
| ON | Hold (hold low>270ms) | OFF |

Must be pulled to ground by load <1K ohms.

External Power (EXT_B+)

Levels:  4.4V0.4V

Loading:  3.0A max